The Satellite Tracking Program SatTrack (V2.0)

====================================================

Manfred Bester, DL5KR

1636 Martin Luther King Jr. Way #L

Berkeley, CA 94709, USA

October 5, 1994

GENERAL OVERVIEW

SatTrack is a satellite tracking program that has been written in 'C'

language on a Sun/UNIX workstation in order to provide an alternative to

PC oriented programs like QuikTrak, InstantTrack, STSplus or OrbiTrack.

It provides two different live displays, for single or multiple satellites,

and two different orbit prediction facilities. Cursor controls used in both

live displays are compatible with the VT100 standard, which allows the

program to be run basically from any terminal. An Xwindows graphics display

will be added in a future release (V3.0). The orbit prediction can be run

either on-line or in a batch mode. In the latter case all necessary

parameters are specified on the command line.

The program reads the NORAD/NASA two-line Keplerian element (TLE) sets

directly and uses these to run the SGP4 propagation model for low-earth

orbit satellites, or alternatively a simplified 'TLE Mean' model for

satellites with an orbital period longer than 225 minutes. The latter

model will be replaced by the SDP4 model in a future release. Other input

files contain information on various default start-up parameters and

satellite modes like beacon frequencies, launch dates, attitudes and

transmission modes. Another file contains the data base for the ground

stations and the nearest city display.

The present version of SatTrack can track a single satellite and/or display

multiple satellites for a single ground station. It also can also control

suitable ground station hardware, like antennas and radio equipment, and

has an autotrack mode in which it switches automatically between a number

of specified satellites. For calibration purposes tracking of the Sun and

the Moon are also provided. There is no software for co-visibility between

several satellites or between several ground stations installed in this

release. Future releases will provide more options. SatTrack requires less

than 1.5 MB of memory at run time.

This program is not to be confused with two other programs, also named

SatTrack, written by different authors on different computers. One, the

Amiga based program, was written by Randy Stackhouse, N4RTL, and the other,

the MacIntosh based program, by Mike Pflueger, WD8KPZ.

INSTALLATION

To install SatTrack on a Sun/UNIX workstation, place the distribution file

'sattrack.V2.0.tar.Z' into your home directory. Uncompress the file by

typing 'uncompress sattrack.V2.0.tar'. Then unbundle the distribution file

by typing 'tar xvpf sattrack.V2.0.tar'. This will create the following

directories in your UNIX home directory:

SatTrack/data

SatTrack/doc

SatTrack/obj

SatTrack/pred

SatTrack/run

SatTrack/src

SatTrack/tle

All source code will be placed in 'SatTrack/src/*' and all auxiliary data

files (see below) in 'SatTrack/data' and in 'SatTrack/tle'. To recompile

SatTrack change into the 'src' directory and type first 'make clean' to

delete all object code and the executables (these files may have been

deleted already for the distribution of SatTrack). Edit the Makefile to

make sure the compile and link flags are properly set for the type of

machine being used. The default set-up is for a Sun-4 (SPARCstation).

The compilation of this software package should run perfectly on at least

Sun-3's (SunOS 4.0.3), SPARCstation IPC's (SunOS 4.1.1) and

SPARCstation 10's (Solaris 1.1 / SunOS 4.1.3) with UNIX BSD systems. These

are the types of machines the program was written and tested on.

Apparently SunOS 4 systems require 'stdlib.h' to be NOT included. Therefore

'SUNOS4' is defined in the Makefile as a compiler flag for both Sun-3's and

Sun-4's.

Certain machines use C compilers different from 'cc'. If, for example,

the default C compiler is 'gcc' (the Gnu C compiler), the user needs to

make sure that 'cc' is linked 'gcc'. This was necessary to compile SatTrack

on a Toshiba T4600C laptop computer running LINUX (a public domain version

of UNIX for PCs). For this machine, the i386 lines for the compile and link

flags (CC_CFL and CC_LFL) have to be activated in the Makefile. The

installation for this particular machine was 'SLACKWARE 2.0' with a

kernel 1.1.50. An older version of the kernel will probably work, too.

A few more compiler flags and other parameters can be configured by the

user. These are defined in 'SatTrack/src/include/sattrack.h'. A future

release will also have an Xwindows graphics display with the typical world

map tracking chart. But in order to allow the program to be used on

machines or terminals without Xwindows capabilities, the line

#define XGRAPHICS

will be used to switch the Xwindows option on or off. If your VT100 type

terminal displays reverse video in an unacceptable way, the line

#define REVERSEVIDEO

in 'sattrack.h' can be commented out before recompilation. This will

suppress reverse video, reverse blink video and underline characters.

If calculations of transits of satellites in front of the Sun are not

needed, the statement

#define SUNTRANSITS

has to be commented out. This will suppress certain menu options.

The precision of the tracking calculations was tried out on the ISI

telescope system (see below in the Tracking Control section). There are

certain interface functions that need to be linked for this purpose.

However, normally these would not be needed. To allow the source code to

be linked without these functions, the statement

#define ISI

has been commented out. Corresponding changes were made to the Makefile

in order to allow straight forward compilation.

In order to finally compile all code, type 'make all'.  Once the

compilation is accomplished the 'run' directory will contain the executable

files and the 'obj' directory all object code. Edit your '.login' and/or

'.cshrc' files to include '$HOME/SatTrack/run' in your path. The 'tle'

directory contains the files with the NORAD/NASA TLE sets and the 'data'

directory the data files 'defaults.dat', 'modes.dat', 'cities.dat', and

'satlist_nnn.dat'. Predictions can be written into an output file with the

name being that of the satellite. It will be placed in the 'pred'

directory.

Previous releases of SatTrack have also been compiled by various users on

different computers. An email collection of patches for machines other than

Sun workstations is provided in the 'doc' directory which contains all

documentation on SatTrack.

INPUT DATA FILES

Program SatTrack requires five input data files, all in plain ASCII format:

tlex.dat         contains NORAD and/or NASA TLE sets

defaults.dat     contains default start-up parameters

cities.dat       contains data on about 800 cities and other special

locations around the globe

modes.dat        contains additional information on satellites (beacon

frequencies, alias names, modes, launch dates etc.)

satlist_nnn.dat  contains a list of satellite names for the multisat

live display ('nnn' denotes the satellite group);

there can be many customized files of this type like:

satlist_all.dat:  all satellites (automatically generated)

satlist_am.dat:   amateur satellites

satlist_com.dat:  communications satellites

satlist_glo.dat:  Glonass navigation satellites

satlist_gor.dat:  Gorizont satellites

satlist_gps.dat:  GPS navigation satellites

satlist_noaa.dat: NOAA weather satellites

satlist_sci.dat:  scientific research satellites

satlist_sts.dat:  space shuttles

satlist_tv.dat:   television satellites

satlist_wx.dat:   weather satellites

Example of a few entries in 'tlex.dat':

Mir

1 16609U 86 17  A 92253.83202629  .00010821  00000-0  17200-3 0  5392

2 16609  51.6235 184.3926 0003511 132.9676 227.1601 15.53877561375443

COBE

1 20322U 89 89  A 92253.56214384 -.00000059  00000-0 -26243-4 0  5574

2 20322  99.0083 266.1778 0008361 270.9185  89.1022 14.03352237143838

PACSAT

1 20439U 90  5  D 92246.03439168  .00000070  00000-0  35288-4 0  4890

2 20439  98.6417 327.8611 0011066 315.7837  44.2460 14.29731036136265

DO-17

1 20440U 90  5  E 92251.71853570  .00000070  00000-0  34955-4 0  4902

2 20440  98.6415 333.6363 0010985 298.5546  61.4530 14.29859354137082

TDRS 4

1 21639U 91 54  B 92249.52615843  .00000111  00000-0  00000+0 0  2232

2 21639   0.0810 282.2753 0002729 241.6567 196.0869  1.00275770  2147

EUVE

1 21987U 92 31  A 92232.33773712  .00002181  00000-0  10297-3 0    77

2 21987  28.4322 199.3801 0010738 262.4472  97.5038 15.15165949 11029

STS-47

1 22120U 92 61  A 92259.61569380 0.00076435  00000-0  25599-3 0   124

2 22120  56.9971  93.6720 0008801 289.1809  70.8314 15.89530661   497

The meaning of the numbers in each line are explained in the source code

listing (see 'satelem.c'). TLE sets like the above can for example be

obtained from 'archive.afit.af.mil' (129.92.1.66) via anonymous FTP.

Change into the directory '/pub/space' and get 'tle.new'. Then place this

file as 'tle.dat' in the directory 'SatTrack/tle' and then run 'tlexgen'.

This procedure can be automated using 'cron' in the following way: use the

command 'crontab -e' to create a crontab file with the following content:

0 15 * * *    $HOME/SatTrack/run/getkeps

This will run 'getkeps' every day at 15:00 h local time (in this example).

The executable C shell script 'getkeps' that calls up 'archive.afit.af.mil'

(129.92.1.66) via anonymous FTP needs to be in the directory 'SatTrack/run'

and contains the following lines:

#! /bin/csh

#

set ftphost=129.92.1.66

set userid=$USER@`hostname`.`domainname`

cd $HOME/SatTrack/tle

#

ftp -n $ftphost << !

user anonymous -$userid

cd /pub/space

get tle.new

get gorizont.tle

get other.tle

get tvro.tle

get visual.tle

quit

!

#

"cp" tle.dat tle.bak

"mv" tle.new tle.dat

"mv" gorizont.tle gorizont.dat

"mv" other.tle debris.dat

"mv" tvro.tle tvro.dat

"mv" visual.tle visual.dat

#

$HOME/SatTrack/run/tlexgen

$HOME/SatTrack/run/selectsat am

#

After placing the data files into 'SatTrack/tle', program 'tlexgen'

is invoked. This program generates an extra file 'tlex.dat' with extended

TLE sets that is basically a concatenation of several individual TLE

files. Duplicate element sets will be filtered out on the basis of the

object number, so that there is ONLY ONE ENTRY PER OBJECT, and this will

be the first element set that is found for that particular object in the

list of TLE files. The file 'tlex.dat' is then used as the default data

base for SatTrack. The names of the data files being concatenated are

specified in the input file 'tlelist.dat' in 'SatTrack/data'. The user

needs to make sure that the order of the input files for 'tlexgen' as

specified in 'tlelist.dat' is chosen correspondingly, if a TLE set for

a particular satellite occurs in more than one file.

The last line in the above shell script invokes program 'selectsat' with

the command line argument 'am' (in this example) that specifies the

amateur satellite group. This will produce the file 'tles.dat' which

contains the TLE sets for those satellites specified in the data file

'satlist_am.dat'. The file 'tles.dat' is not used with SatTrack, but is

useful for distribution to other users in the community who are interested

only in a particular subset of all TLE sets, in this example those of

amateur satellites. The C shell script 'getkeps' can also be invoked with

the '-v' option.

The TLE sets can also be obtained from the Celestial Bulletin Board System

in Fairborn, Ohio, via modem line (513) 427-0674. Latest element sets for

space shuttle missions can be obtained from 'spacelink.msfc.nasa.gov'

(192.149.89.61) via TELNET or via telephone by dialing (205) 895-0028, or

from an automatic mail service (see 'SatTrack/doc/info.tle_shuttle'). An

element set for STS-47, for example, would be placed in 'SatTrack/tle'

as 'sts-47.dat'.

Example of 'defaults.dat':

Site:             Berkeley, CA, USA

Callsign:         DL5KR

Satellite:        16609

Satellite group:  am

Element set TLE:  tlex.dat

Element set STS:  sts.dat

Element set type: NASA

Time zone:        PDT

Duration:         6.0

Min elevation:    0.0

Tracking:         0

Antenna:          /dev/ttyp1

Radio:            /dev/ttyp4

Display lines:    24

X display lines:  64

X terminal type:  xterm

Pressure:         725.0

Temperature:      15.0

Humidity:         35.0

Amongst these parameters are the defaults that show up at the menu in <>

brackets when SatTrack is started. The satellite can be specified either

with its name or with the object number. However, if the name is

ambiguous (as in 'Mir' and 'MIR DEB') the object number needs to be

specified. If a satellite group 'am' is specified as the default group

for the multisat live display, then there needs to be a corresponding file

called 'satlist_am.dat' with the names of the satellites in it.

The next three lines deal with the two-line element sets. At program start

SatTrack reads the entire default data file, here specified as 'tlex.dat',

and stores it in memory. If another data file is specified at the menu line

the program will add all satellites from that file to the memory list, as

long as the maximum number of satellites as specified in 'sattrack.h' is

not exceeded. Data for satellites that are in the list already will not be

read again. For space shuttle data sets it is important to specify the type

of the element set, mainly because of the convention for the orbit number.

The two possible options are 'NASA' and 'NORAD'.

Options for the time zone are UTC, CET, MEZ, MSZ, EEST, EEDT, EST, EDT,

CST, CDT, MST, MDT, PST, PDT, and HST. If other time zones are needed

these will have to be added in 'satstrings.h'. In case a time zone is

entered which is not in this list the time zone is automatically set to

UTC. Alternatively, a time zone can be entered as the numerical value. In

this case the local time is displayed as 'LOC'. SatTrack does not

automatically check for daylight savings time when reading the system

time, since it always reads the UTC system time, and the change over to

and from daylight savings time depends on the location of the ground

station. Therefore the default start-up parameter for the time zone needs

be edited by hand accordingly. All calculations are performed in UTC, of

course. Other time zones are used for convenience only in the live displays

and in the prediction tables.

The entries 'Tracking', 'Antenna' and 'Radio' are used for controlling the

antenna and radio interfaces. The default value for the tracking flag can

be set to '0' or '1'. If set to '1', the tracking will be turned on

automatically at program start, and otherwise it will be off. The two lines

'Antenna' and 'Radio' contain the information as to which interfaces the

radio and the antenna controllers are connected.

The following three lines deal with the type of terminal or window used for

the live displays. If the environmental variable 'term' matches the entry

in the line 'X terminal type' it is assumed that the terminal or window can

display 132 characters and the number of lines specified in 'X display

lines'. Otherwise, the width of the terminal or window is assumed to be 80

characters and the number of lines as specified in 'Display lines'. The

entries in the three lines 'Pressure' [mmHg], 'Temperature' [C] and

'Humidity' [%] are used to calculate the refraction in the atmosphere for

tracking satellites visually with an optical telescope.

Example of 'cities.dat':

`North Pole`,90,0,0

`South Pole`,-90,0,2800

`Abeche, Chad`,14.46,-20.68,0

`Aberdeen, SD, USA`,45.467,98.483,0

`Abilene, TX, USA`,32.465,99.713,0

`Abu Dhabi, United Arab Emirates`,24.47,-54.37,0

`Acapulco, Mexico`,16.85,99.92,0

`Accra, Ghana`,5.6,0.167,0

`Adamstown, Pitcairn Island (UK)`,-25.07,130.08,0

`Addis Ababa, Ethiopia`,9,-38.733,0

This example shows the first 10 entries in 'cities.dat'. The first two

lines, i.e. the North and South Pole, have to remain unchanged. The name

of the location has to be included in `` and can be up to 37 characters

long. The following numbers are latitude [deg North], longitude [deg West],

and altitude [m], all separated by commas. Note that some other tracking

programs count the longitude positive in eastern direction. Here the

convention of positive signs for western longitudes is used. On the other

hand, SatTrack counts the time zone negative in western direction. More

locations can be added to 'cities.dat', if desired. Note that the maximum

number of entries allowed is presently 800. If more entries are required

the variable MAXCITIES in 'sattrack.h' needs to be changed accordingly and

the source code needs to be recompiled.

Data on cities in the United States can be obtained from a geographic data

base via 'telnet martini.eecs.umich.edu 3000' (telnet 141.212.99.9 3000).

Example of 'modes.dat':

Satellite: OSCAR 10

Object:    14129

Alias:     AO-10

Beacon:    145.810 MHz

Mode:      B  0 - 256

Satellite: UoSat 2

Object:    14781

Alias:     UO-11

Beacon:    145.826 MHz

Mode:      FM  0 - 256

Satellite: AO-13

Object:    19216

Alias:     Phase 3C

Beacon:    145.812 MHz

Attitude:  230.0 -5.0 deg

Mode:      B     0 - 170

Mode:      BS  170 - 218

Mode:      Sb  218 - 220

Mode:      St  220 - 230

Mode:      BS  230 - 250

Mode:      B   250 - 256

Satellite: RS-12/13

Object:    21089

Beacon:    145.912500 MHz

Satellite: Mir

Object:    16609

Beacon:    145.550 MHz

Mode:      FM  0 - 256

Satellite: STS-55

Object:    22640

Alias:     Columbia

Launch:    26Apr93  14:50:00 UTC

Beacon:    145.550 MHz

Mode:      FM  0 - 256

Satellite: HST

Object:    20580

Alias:     Hubble

Satellite: NOAA-12

Object:    21263

Beacon:    137.500 MHz

Mode:      FM  0 - 256

Note that if a satellite is listed in 'modes.dat' its name in the line

'Satellite:' has to exactly match the name in the NORAD TLE data file with

lower and upper case letters. For example, AO-10 has to be entered as

OSCAR 10 and AO-16 as PACSAT. An alias name with a maximum length of 15

characters can be specified in the 'Alias:' line. This name will be used

in addition to the NORAD satellite name for the live display and the

prediction tables. The specification of the object number is needed to

ensure the proper assignment of parameters, in case there are several

matches for the satellite name, like 'Mir' and 'MIR DEB'. If the satellite

to be tracked is not listed in 'modes.dat' or if no beacon frequency is

specified, the program will use a default value of 146.000 MHz. If a

launch date is specified in 'modes.dat' the live display will show the

mission elapsed time (MET). The prediction tables will show the MET for

the space shuttle only.

Example of 'satlist_am.dat':

16609

OSCAR 10

AO-13

PACSAT

WO-18

LO-19

DO-17

FO-20

AO-21

UO-22

KO-23

KO-25

IO-26

AO-27

PO-28

RS-10/11

RS-12/13

Note that the satellites' names need to match those in the TLE data file.

If there are ambiguous entries in the TLE file, like 'Mir' and 'MIR DEB',

the object number needs to be specified instead of the name, like '16609'

for Mir. In case of the space shuttle the correct specification of the

object number is rather important since there are pre-launch TLE sets with

a preliminary object number. For instance, in the case of STS-64, the

object numbers used were '64' or '99964' for the pre-launch element sets,

and '23251' for the post-launch element sets. It is VERY IMPORTANT that

the object number matches in all resource files. In case of STS-64 these

are the three files 'sts-64.dat', 'satlist_am.dat' (and for that matter

all other files of the type 'satlist_nnn.dat' that contain STS-64's object

number) and 'modes.dat'. After launch, when the new post-launch object

number is issued, all these resource files need to be edited

correspondingly.

PROGRAM START

To start the program type 'sattrack [-b] [-d] [-m] [-q] [-t] [-v]'.

Remark: It is possible to combine several options. However, it is NOT

POSSIBLE to combine them in one argument, like 'sattrack -vtq'. The

command line options are as follows:

-b   enables the BATCH mode for orbit predictions. All parameters have to

be entered on the command line.

-d   enables the DEBUG mode. In this case various intermediate results

will be displayed.

-m   performs all calculations with the simplified MEAN Keplerian

propagation model for both low-earth and deep space objects.

As long as the SDP4 model is not installed yet, the simplified model

will be automatically used for all deep space objects.

-q   starts up SatTrack QUICKLY into the multisat live display, with all

parameters as specified in the resource file 'defaults.dat' --- no

questions asked.

-t   allows specification of another TIME zone instead of the default time

zone as listed in 'defaults.dat'. This is useful to quickly run off a

prediction for a site anywhere in the world without having to change

the default data file.

-v   enables the VERBOSE mode. All default parameters, ground station data

and detailed orbital elements will be displayed at program start.

For all keyboard entries the default parameters are shown in <>. Hitting

<CR> each time will use these. Otherwise, both lower and upper case

letters will be accepted. When entering the site and the satellite names a

fraction of the name will be sufficient, as long as they are unambiguous.

Otherwise all ambiguities will be displayed.

The following chapters describe the two different live displays and the

two different orbit prediction features.

MAIN MENU

Upon program start the user is asked to enter the time zone (only if

SatTrack is invoked with the '-t' option), the ground station name, the

name or object number of a preferred satellite, the name of the file with

the TLE sets, and in case the preferred satellite is one of the space

shuttles, the type of the TLE set (NASA or NORAD). Then the main menu is

presented with the following options:

Live Display     : D

Orbit Prediction : P

Restart          : R

Quit             : Q

State Vector Test: T  (not shown as a menu option)

The state vector test allows verification of the calculations with the

SGP4 and SDP4 models against the numerical examples given in the

Spacetrack Report (see source code of 'sattest.c' for exact reference).

The TLE sets needed for these tests are in 'SatTrack/tle/test.dat'. For

testing the SGP4 model specify the object number as '40' and the TLE file

name as 'test.dat'. For testing the SDP4 model, the object number needs to

be specified as '41' (once the SDP4 model is installed).

LIVE DISPLAY FOR SINGLE AND MULTIPLE SATELLITES

In the live display mode the user can choose between a simultaneous

display for a number of satellites with a shorter list of tracking

parameters, i.e. one line per satellite, or a single satellite with the

full set of tracking parameters, i.e. the full screen. In either case

both the antenna and the radio equipment can be controlled. In the multiple

satellite display the cursor ">" can be moved to choose the satellite

being tracked. The default cursor position is on the line of the satellite

that was chosen at the main menu. If that satellite is not in the

specified group list it will be added to it. The keyboard commands for the

multisat live display are also allowed in the single satellite display,

although those that change the cursor position will not be effective. The

allowed single key commands are:

t: toggles tracking "On" and "Off"

T: toggles tracking between "Auto" and "On"

a: moves cursor automatically to satellite with the next AOS time

(same with "A")

d: moves cursor down one line

D: moves cursor down 10 lines (if less than 10 lines, to bottom of list)

u: moves cursor up one line

U: moves cursor up 10 lines (if less than 10 lines, to top of list)

j: moves one page down (if satellite list is longer than one page)

J: moves to bottom page

k: moves one page up (if satellite list is longer than one page)

K: moves to top page

l: adds or deletes satellite to/from autotrack list (marked "-" or ">")

L: adds or deletes all displayed satellites to/from the autotrack list

n: move cursor to a new satellite by name (or fraction thereof)

(same with "N")

o: move cursor to a new satellite by object number (same with "O")

s: changes instantaneously from multisat to single satellite live display

(same with "S") (also refreshes single satellite display)

m: changes instantaneously from single satellite to multisat live display

(same with "M") (also refreshes multisat live display)

q: quit SatTrack (back to UNIX prompt) (same with "Q")

0: (zero) enables tracking of the active satellite (in the single

satellite live display only, turns off tracking of the Sun or the Moon)

1: enables tracking of the Sun (in the single satellite live display only)

2: enables tracking of the Moon (in the single satellite live display only)

CR, LF, ESC: interrupt either live display (go back to main menu)

Depending upon the type of terminal used, the multisat live display can

show the orbit prediction model used (the 'M' column), and the ground track

location. This is the case if the terminal is an Xwindows terminal

(assuming that the window in which the program is started has been set to

a width of 132 characters). Otherwise this part of the live display is

omitted. This feature depends upon the setting of the default parameter

'X display type' in the 'defaults.dat' file. If the entry chosen is for

example 'xterm' and the user logs in on an Xwindows terminal, the program

assumes that the window is set to a size of 132x64 characters. The default

length (i.e. number of lines) can also be set in 'defaults.dat'. Satellites

are shown in the order of the time of AOS if they are below the horizon, or

in reverse order of the time of LOS if they are above the horizon. An

asterisk next to the maximum elevation (MEL) of the next pass indicates a

'good' pass, i.e. one with an elevation of more than 40 deg. The symbol to

the right of the instantaneous satellite elevation indicates, if the

satellite is currently ascending or descending. The letter in the 'M'

column shows the type of propagation model used for that satellite. The

three different models used are the SGP4 model ('G'), the SDP4 model ('D'),

or the simplified TLE Mean model ('M'). The latter model is automatically

used for all satellites when SatTrack is invoked with the '-m' flag.

Otherwise SatTrack chooses between the SGP4 and SDP4 model, depending upon

the satellite's orbital period. However, in the present release the SDP4

model is not yet installed and SatTrack always uses the TLE Mean model for

deep space objects.

The multisat live display can be very slow if the computer is not of a

SPARCstation quality and if the number of satellites chosen is large. In

this case the computing power will not be sufficient to ensure timely

control of the antenna or the radio. SatTrack has to find the circumstances

of the next pass for each satellite before the first display update occurs.

In the live display mode for single satellites the program cycles through

a full computation every 5 or 10 seconds and sleeps in between. This time

can be changed, of course, by setting the variables FASTUPDATEINT,

MEDIUMUPDATEINT and SLOWUPDATEINT in 'sattrack.h'. Whenever the environment

variable 'term' corresponds to the default parameter 'X display type' in

'defaults.dat', the fast update is chosen. This allows the user to login

on a fast console and have the display updated with the fast update time,

whereas on a 9600 Bd type terminal the slow update time is used for the

multisat display and the medium one still for the single satellite display.

In a strict sense, the live displays are not real-time displays since the

program runs under UNIX. For most applications, however, this is not a

disadvantage. For near real-time performance it is important that the

UNIX system clock is set correctly, though. Examples of the screen

layouts for both live displays are shown below.

The singlesat live display is rather self-explanatory. The top center of

the display shows the date and time, in the specified time zone.

In case of the space shuttle, the computation of the orbit number is

different from all other satellites. For the shuttle, the orbit

number increments each time it crosses the ascending node. The shuttle is

launched into orbit #1. If the space shuttle is tracked, the source of the

element set, i.e. the element set type has to be specified, since there is

a difference in the convention for definition of the reference orbit

between NASA and NORAD. For satellites other than the space shuttle the

NORAD convention is used and the orbit number increments when the satellite

passes through the perigee. Note that in case of the space shuttle, the

results of the live calculations are displayed only if the current time is

past the epoch time of the element set used. If the current time is between

the launch time and the epoch time of the element set, the display will

show 'Ascending to Orbit'.

The visibility display to the right of the percentage of the current orbit

indicates if the satellite is in the day ('D') or night ('N') part of its

orbit. If the satellite is above the horizon and in the day part of its

orbit and at the same time the Sun is below the nautical twilight limit

(-6 deg elevation), then the display shows 'V' (visible), since the

satellite will be visible "to the naked eye", at least for the larger

satellites. However, the apparent brightness depends on the relative

angles between the Sun, the satellite and the ground station, and on the

shape, size, attitude and distance (range) of the satellite.

The eclipse and visibility conditions are also indicated by 'D', 'N' or

'V' in the lower right corner of the display for the circumstances of the

next pass, i.e. acquisition of signal (AOS), maximum elevation (MEL) and

loss of signal (LOS) of the satellite.

Azimuth and elevation angles of the Sun and the Moon are shown for the

location of the ground station. The elevation angles for both the Sun, the

Moon and the satellite are corrected for refraction in the Earth's

atmosphere using the atmospheric parameters in 'defaults.dat'. This

facilitates observations of satellites with an optical telescope.

The Doppler shift is calculated for the beacon frequency displayed one

line above. The free-space path loss can be used for link quality

calculations.

The satellite mode depends on the phase of the satellite (the mean anomaly

in units of 0-256), as specified in 'modes.dat'. If no satellite mode is

specified, the display shows blank.

The letters to the right of the elevation and latitude display indicate

if the satellite is on the ascending ('/') or descending ('\') part

of its pass or orbit, respectively.

In case of the space shuttle, distances are displayed in nautical miles

(nm), and otherwise they are in kilometers. Accordingly, velocities are

in nm/s or km/s.

The state vector of the satellite (position and velocity in Cartesian

coordinates) is displayed below the bearing and ground track information.

The circumstances of the next pass are calculated and displayed in the

lower part of the screen. If the next pass has a maximum elevation of more

than 40 deg the maximum elevation and range at maximum elevation are shown

in reverse video, since this would be considered a "good pass". The

countdown time represents the time until next AOS or next LOS. One minute

before AOS and during a satellite pass the countdown time is displayed in

reverse video.

Rise and set times are not calculated for geostationary satellites.

However, one needs to make some kind of distinction between geosynchronous

and geostationary satellites. SatTrack uses two parameters for this

purpose, a limit on the mean motion and a limit on the inclination. If the

mean motion is 1.0 +- 0.05 rev/day the satellite is considered

geosynchronous, and if in addition the inclination is less than 10 degrees,

it is also considered geostationary. Both parameters, GEOSYNCMOT and

GEOSTATINC, are defined in the include file 'sattrack.h'. They can be changed before

recompilation, if necessary. There are quite a few satellites with

inclinations in the range below 10 degrees. If for instance the ground

station is in North America, some of these will never set. On the other

hand, if the ground station is at the south pole, they will rise and set.

Specifying the inclination limit is therefore important for the algorithm

that searches for the circumstances of the next pass. The time limits for

rise and set time searches are specified in 'sattrack.h' by the parameters

MAXDAYS and MAXDAYSSYNC for non-geosynchronous and geosynchronous

satellites, respectively.

The mission elapsed time (MET) will be displayed if the launch date and

time are specified in 'modes.dat'.

The ground track location indicates the location of the sub-satellite

point with respect to the nearest location in the city data base

('cities.dat').

An alarm (beep) is generated for each screen update within one minute of

either AOS or LOS. When the satellite being tracked is above the horizon

several numbers on the screen show up in reverse video (the header line,

azimuth, elevation, range, Doppler shift and path loss).

Once the program is running in the live display mode, the screen can be

frozen and unfrozen with ^S and ^Q, respectively.

TRACKING CONTROL

The tracking feature can be used in both live displays. Tracking is

automatically turned off when the cursor is moved from one satellite to

another in the multisat display, but stays active when switching back and

forth between the two live displays. When the tracking is enabled the

satellite orbit calculations will be performed for a time ahead of the

present. The time offset and also the interval at which the calculations

are repeated are specified by the variables TRACKINT and TRACKTIMEAHED

in 'sattrack.h'. Default values are 1 and 10 seconds, respectively. If the

computer that performs both the calculations and the live display does

also the controlling of the ground station hardware, the parameter

TRACKTIMEAHEAD needs to be set to zero. Even though the live display may

only be updated every 5, 10 or 15 seconds, the tracking calculations and

control commands are still performed every TRACKINT seconds.

If the tracking calculations and control of ground station equipment are

performed on different machines, the calculations may have to be

accomplished several seconds ahead of time to allow for transfer of the

tracking information to the second computer and therefore timely antenna

and radio tracking. This is the case with the Infrared Spatial

Interferometer (ISI), a two-telescope infrared telescope system, on which

this software was tried out. This system has its own antenna/telescope

control computers with the vxWorks real-time operating system and receives

the tracking information from a Sun workstation once per second and 10

seconds ahead of time via remote procedure calls (RPC) on the Ethernet.

The particular interface code for these RPCs can be found in 'sattele.c'.

If the variable 'ISI' is undefined in 'sattrack.h', the RPC interface

functions will be inactive and the source code can be compiled on machines

that do not have the interface code for the ISI system.

In the autotrack mode, the program will automatically switch between

those satellites in the multisat live display that have been marked to

be in the autotrack list ('>' or '-'). Once the currently tracked

satellite passes below the horizon, the program automatically switches to

the next satellite in the list, i.e. the one with the next set time, or,

if none of the marked satellites is above the horizon, the one with the

nearest rise time. Satellites can be marked or cancelled from the

autotrack list using the 'l' or 'L' commands (see above). The autotrack

mode works only while in the multisat live display. If the user switches

to the single satellite live display while autotracking is enabled, the

program will not switch satellites.

The devices to which the antenna and radio interfaces are connected is

specified in 'defaults.dat'. One can specify devices like '/dev/ttyp1'

and '/dev/ttyp2', for example, to direct the tracking commands to some

'xterm' windows. To control particular ground station equipment the program

code needs to be adapted by the user (see 'satctrl.c' and 'sattrack.h').

It should be noted that the tracking will not work right if the computer

is overloaded. If, for instance the multisat live display is used with

500 satellites (specify satellite group as 'all'), even a SPARCstation IPC

will not be able to perform the tracking calculations on time. The number

of satellites in the multisat live display should be kept small, i.e. of

the order 20, to insure that the tracking calculations can be performed on

time.

ORBIT PREDICTIONS

The orbit prediction mode allows the calculation of satellite passes for

a given ground station. The data will be either displayed on the video

terminal or written into a file in the 'pred' directory which can

later be obtained as a hardcopy. The program generates tables in a short

and in a long form. The short form listing consists of one table per day

with one line per pass, indicating the time at AOS, MEL and LOS, the

duration of the pass, the azimuth at AOS, MEL and LOS, the maximum

elevation of the pass, the visibility code for AOS, MEL and LOS, and

number of the orbit in which the satellite is at AOS. Also, for the space

shuttle, if a launch date is specified in 'modes.dat' an additional column

shows the MET at AOS. Passes with a maximum elevation of more than 40 deg

are marked with an asterisk next to the MEL value.

The prediction in the long format consists of 9 columns. The first column

is the time in specified fixed increments from line to line. Most of the

table is self-explanatory. The column labeled 'Loss' is the free-space

path loss in dB. The column labeled 'Phs' is the orbital phase of the

satellite, i.e. the mean anomaly running from 0 through 256. The 'Md'

column is the satellite mode, derived from the phase and the entries in

'modes.dat'. This column will be blank if no modes are specified in

'modes.dat'. The last column is the visibility code of the satellite ('D',

'N' or 'V', see above).

A special feature of the orbit prediction in the long format is the

computation of transits of a given satellite in front of the solar disk.

In this case the output format of the prediction table is different.

An example for prediction tables in the short and long forms, and for

solar transits are given below.

The orbit prediction in both short and long form can also be run in the

batch mode. All parameters have to be entered on the command line with

a fixed format. An example for shell scripts that run SatTrack in the

batch mode for a number of satellites are given in 'SatTrack/run' as

'passes' or 'transits', for the prediction of passes over a specified

ground station or transits in front of the Sun, respectively.

PRECISION

The results from SatTrack, namely the latitude and longitude of the

sub-satellite point and the azimuth and elevation angles of a satellite

were compared with those calculated with QuikTrak, InstantTrack, STSplus

and MacSat. It was found that all programs agree with each other within

a few tens of a degree. The precision obtained depends amongst other

things on the particular model of the Earth used. SatTrack uses a

non-spherical geoid model. Also, the precision depends on the level of

sophistication used for the astronomical coordinate transformations, that

is the motion of the Earth within the solar system. Of course, the

precision also depends on the quality of the set of orbital elements for

the orbit simulation and the level at which the periodic variations of the

SGP4 and SDP4 orbital models are taken into account. The NORAD TLE sets

are mean values with periodic variations removed in a particular way. For

high-precision tracking, these variations have to be reintroduced by the

tracking program in the same way they were removed, depending upon what

model was used. This current version of SatTrack does reintroduce these

periodic variations only for low-earth objects, i.e. those with an orbital

period of 225 minutes or less. This is accomplished by using the SGP4

propagation model. For deep space objects, i.e. those with an orbital

period longer than 225 minutes, a simpler propagation model, called the

'TLE Mean Model' is presently used. However, the precision of SatTrack

should be high enough to properly steer antennas with a gain of 30 dBi or

less (beam width of 6.5 deg or larger).

The precision of the calculation of the positions of the Sun and the Moon

were tested on May 10, 1994 with the annular solar eclipse for a ground

station in El Paso, Texas, which was in the center of the shadow path. It

was found that the differential angle between the Sun and the Moon was

smaller than 1.5 arcmin or 1/20 of the diameter of either object. This

suggests that the absolute precision is about half that value, or

45 arcsec. Furthermore, tracking of the Moon was also checked with an

astronomical telescope, namely one of the 1.65 m telescopes of the Infrared

Spatial Interferometer (ISI) at Mt. Wilson Observatory. It was found that

the precision again is of the order 45 arcsec.

It is also planned to check the precision by visual tracking of various

low-earth orbit and geostationary satellites.

AUXILIARY PROGRAMS

Three of the following four programs are not used with SatTrack, but are

rather useful for other purposes.

amsat          generates the data file 'amsat.dat' in 'SatTrack/data' with

orbital elements in the AMSAT format from a data file with

orbital elements in the NORAD two-line format.

checksum       generates two-line element sets with the old NORAD checksum

convention needed with InstantTrack V1.0. The input file

is for example 'tle.dat' and the corresponding output file

is 'tle.chk', also created in 'SatTrack/tle'.

selectsat [-v] generates a data file 'tles.dat' with a selected number of

TLE sets from 'tle.dat'. The names of the satellites to be

selected are read from 'satlist_nnn.dat', where 'nnn' needs

to be one of the satellite groups for which a satlist file

exists (e.g. 'am', 'com', 'glo', 'gps', 'sci', 'wx', 'all'

or anything supplied by the user).

tlexgen [-v]   concatenates several files with two-line Keplerian element

sets into one file, 'tlex.dat'. Duplicate entries are

filtered out, using the object numbers as a the designator

rather than the satellite names. The TLE input files and the

order in which they are processed are specified in

'SatTrack/data' in the file 'tlelist.dat'. This program also

produces two other output files, namely 'satlist_all.dat'

with the object numbers of all satellites in 'tlex.dat', and

'index.dat' which serves as a reference between object

numbers and satellite names. The latter file is not used by

any program, but is rather meant to be a user friendly

look-up table.

TLEXGEN HAS TO BE RUN WHENEVER ANY OF THE TLE FILES HAVE

BEEN UPDATED OR EDITED.

BUGS AND IMPERFECTIONS

There are no known bugs in SatTrack at this point. However, if bugs are

discovered after the V2.0 release, those will be fixed in a subsequent

version of SatTrack. It seems to be difficult to install this software on

UNIX machines other than Sun workstations, because some of the libraries

have different names or are bundled differently. There also seem to be

various problems with the compilation on other types of machines or with

different versions of UNIX. The author would appreciate to be made aware of

bugs or imperfections discovered by any users. In this case, or if you have

any suggestions, please send a note to 'manfred@sunspot.ssl.berkeley.edu'.

ACKNOWLEDGEMENTS

The author wishes to thank a number of individuals, in particular

U. Watson, C. Degiacomi, W. Marchant (KC6ROL), R. Engberg (K0MVL),

R. Kupiec (N3MML), R. Frohne (KL7NA), D. Brown, G. Clark II (N5VMF),

B. Jones, K. Yamada (JR1EDE) and PE1CHL for suggestions and/or for fixes

with problems of compilation of this software on various machines. Also

many thanks to T. S. Kelso and the JSC/AMSAT SAREX team who make available

the two-line Keplerian element sets. Without these input data SatTrack

would be completely useless.

COPYRIGHT

Copyright (c) 1992, 1993, 1994 Manfred Bester. All Rights Reserved.

Permission to use, copy, modify, and distribute this software and its

documentation for educational, research and non-profit purposes, without

fee, and without a written agreement is hereby granted, provided that the

above copyright notice and the following three paragraphs appear in all

copies.

Permission to incorporate this software into commercial products may be

obtained from the author at the address given above.

IN NO EVENT SHALL THE AUTHOR BE LIABLE TO ANY PARTY FOR DIRECT, INDIRECT,

SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OF

THIS SOFTWARE AND ITS DOCUMENTATION, EVEN IF THE AUTHOR HAS BEEN ADVISED

OF THE POSSIBILITY OF SUCH DAMAGE.

THE AUTHOR SPECIFICALLY DISCLAIMS ANY WARRANTIES, INCLUDING, BUT NOT

LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE. THE SOFTWARE PROVIDED HEREUNDER IS ON AN "AS IS"

BASIS, AND THE AUTHOR HAS NO OBLIGATIONS TO PROVIDE MAINTENANCE, SUPPORT,

UPDATES, ENHANCEMENTS, OR MODIFICATIONS.

EXAMPLE OF THE SINGLE SATELLITE LIVE DISPLAY

(not included)

EXAMPLE OF THE MULTI SATELLITE LIVE DISPLAY

(not included)

EXAMPLE OF THE PREDICTION TABLES IN SHORT FORM

(not included)

EXAMPLE OF THE PREDICTION TABLES IN LONG FORM

(not included)

EXAMPLE OF THE PREDICTION TABLES FOR SOLAR TRANSITS

(not included)