The Satellite Tracking Program SatTrack V1.1

==================================================

Manfred Bester, DL5KR

January 7, 1993

GENERAL OVERVIEW

SatTrack is a satellite tracking program that has been written in 'C'

code on a Sun/UNIX workstation in order to provide an alternative to PC

oriented programs like InstantTrack or QuikTrak. SatTrack is based on

program 'Orbit' by Robert W. Berger (N3EMO), but has been completely

re-written and enhanced with many new features. It provides a live display

which Orbit does not have, and other orbit prediction facilities, but no

graphics either. Cursor controls used in the live display are compatible

with the VT100 standard. The program reads the NORAD two-line element sets

directly. Other input files are defaults.dat, sites.dat, cities.dat and

modes.dat. These contain information on default parameters and various

sites and satellite modes like beacon frequencies, launch dates,

attitudes, transmission modes and so on. The file cities.dat contains the

data base for the nearest city display. SatTrack V1.0 can track a single

satellite for a single ground station. There is no software for satellite

co-visibility between several satellites or between several ground stations

installed.

INSTALLATION

To install SatTrack on a Sun workstation, create the following

directories in your UNIX home directory:

SatTrack/Data

SatTrack/Doc

SatTrack/Object

SatTrack/Prediction

SatTrack/Run

SatTrack/Source

Move all source code into SatTrack/Source. Move all auxiliary data

files (see below) into SatTrack/Data. This is all done automatically if

you uncompress and un-tar the distribution file 'sattrack.tar.Z' in your

home directory.

Edit the file sattrack.h and change the variable STATIONCALL to your

call sign. Change to directory SatTrack/Source and type 'make clean' and

'make all'. Edit your .login and/or .cshrc files to include ~/SatTrack/Run

in your path.

The Run directory will contain the executable files and the Object

directory the object code. The Data directory contains the files

with the NORAD two-line element sets as well as defaults.dat, modes.dat,

cities.dat and sites.dat. Predictions can be written into an output file

which will be placed in the Prediction directory. The Doc directory will

contain all documentation on SatTrack.

INPUT DATA FILES

Program SatTrack requires five input data files, all in ASCII format:

tle.dat          contains NORAD or NASA two-line element sets

defaults.dat     contains default start-up parameters

sites.dat        contains the data base with ground station parameters

(name, latitude, longitude, altitude)

cities.dat       contains data on nnn cities and other special locations;

this part is not finished yet; one location (KSC)

is presently hardwired in satcity.c

modes.dat        contains additional information on satellites (beacon

frequencies, alias names, modes, launch dates etc.)

Example of tle.dat:

- Current Two-Line Element Sets #053 -

Mir

1 16609U 86 17  A 92253.83202629  .00010821  00000-0  17200-3 0  5392

2 16609  51.6235 184.3926 0003511 132.9676 227.1601 15.53877561375443

COBE

1 20322U 89 89  A 92253.56214384 -.00000059  00000-0 -26243-4 0  5574

2 20322  99.0083 266.1778 0008361 270.9185  89.1022 14.03352237143838

PACSAT

1 20439U 90  5  D 92246.03439168  .00000070  00000-0  35288-4 0  4890

2 20439  98.6417 327.8611 0011066 315.7837  44.2460 14.29731036136265

DO-17

1 20440U 90  5  E 92251.71853570  .00000070  00000-0  34955-4 0  4902

2 20440  98.6415 333.6363 0010985 298.5546  61.4530 14.29859354137082

TDRS 4

1 21639U 91 54  B 92249.52615843  .00000111  00000-0  00000+0 0  2232

2 21639   0.0810 282.2753 0002729 241.6567 196.0869  1.00275770  2147

EUVE

1 21987U 92 31  A 92232.33773712  .00002181  00000-0  10297-3 0    77

2 21987  28.4322 199.3801 0010738 262.4472  97.5038 15.15165949 11029

STS-47

1 22120U 92 61  A 92259.61569380 0.00076435  00000-0  25599-3 0   124

2 22120  56.9971  93.6720 0008801 289.1809  70.8314 15.89530661   497

The meaning of the numbers in each line are explained in the source

code listing (see 'readnorad.c'). Elements sets like the above can be

obtained from 'archive.afit.af.mil' (129.92.1.66) via anonymous FTP.

Change into the directory '/pub/space' and get 'tle.new'. Then place this

file as 'tle.dat' in the directory SatTrack/Data. This procedure can be

automated in the following way. Use the command 'crontab -e' to create a

crontab file with the following content:

0 15 * * *    /usr1/LOGIN_NAME/SatTrack/Run/getElementSets

This will run 'getElementSets' every day at 15:00 h local time (in this

example). The executable shell script 'getElementSets' that calls up

'archive.afit.af.mil' via anonymous FTP needs to be in the directory

SatTrack/Run and contains the following nine lines:

cd /usr1/LOGIN_NAME/SatTrack/Data

ftp -n 129.92.1.66 << !

user anonymous LOGIN_NAME@HOST.LOCATION.NETWORK

cd /pub/space

get tle.new

quit

!

cp tle.dat tle.bak

cp tle.new tle.dat

LOGIN_NAME@HOST.INSTITUTION.NETWORK, your e-mail address, is the password

used. The two-line element sets can also be obtained from the Celestial

Bulletin Board System in Fairborn, Ohio, via modem line (513) 427-0674.

Latest element sets for space shuttle missions can be obtained from

'spacelink.msfc.nasa.gov' (192.149.89.61) via TELNET or via telephone by

dialing (205) 895-0028. An element set for STS-47, for example, would be

placed in SatTrack/Data as 'sts-47.dat'.

Example of defaults.dat:

Site:             Berkeley CA

Satellite:        Mir

Element set:      tle

Element set type: NORAD

Time zone:        PST

Duration:         3.0

Min elevation:    10.0

These parameters are the defaults that show up in '<>' brackets when

starting program 'sattrack'. Options for the time zone are UTC, MEZ, MSZ,

EST, EDT, CST, CDT, MST, MDT, PST,  PDT, and  HST. If other time zones

are needed these will have to be added in 'sattrack.c'. If a time zone

is entered that is not in this list the time zone is automatically set to

UTC. Program 'sattrack' does not check for daylight savings time when

reading the system time, since the UTC system time is always used.

Example of sites.dat:

Auckland NZL            -36.000000 -174.500000     0.0

Berkeley CA              37.873931  122.240372   400.0

KSC FL                   28.475000   80.610000     5.0

Edwards AFB              34.940000  117.860000   200.0

Guam                     13.330000 -144.720000     5.0

Haleakala AMOS           20.708333  156.257500  3052.0

Honolulu HI              21.310000  157.850000     0.0

Houston JSC TX           29.875000   95.420000    50.0

Mauna Kea HI             19.826667  155.471667  4215.0

Midway Island            28.360000  177.380000     5.0

Moscow RUS               55.700000  -37.545000   195.0

Rio De Janeiro          -22.895000   43.223333    33.0

Washington DC            38.921667   77.066667    92.0

The length of the name of the ground station has to be 15 characters or

less. Otherwise it will be truncated to a length of 15 characters. The

first column of numbers is the geographical latitude [deg North], the

second the geographical longitude [deg West], and the third the altitude

[m]. Note that some other tracking programs count the longitude in eastern

direction. Here the convention of positive signs for western longitudes is

used.

Example of cities.dat:

--- not yet required ---     (a city data base has not yet been located)

Example of modes.dat:

Satellite: OSCAR 10

Alias: AO-10

Beacon:          145.810000 MHz

Mode: B           000 - 256

Satellite: AO-13

Beacon:          145.812000 MHz

Attitude:        180.0  0.0 deg

Mode: B           000 - 100

Mode: JL          100 - 120

Mode: LS          120 - 135

Mode: JL          135 - 150

Mode: B           150 - 256

Satellite: PACSAT

Alias: AO-16

Beacon:          437.026250 MHz

Satellite: INFORMTR-1

Alias: AO-21

Beacon:          145.822000 MHz

Satellite: Mir

Beacon:          145.550000 MHz

Mode: FM            0 - 256

Satellite: STS-47

Alias: Endeavour

Beacon:          145.550000 MHz

Mode: FM            0 - 256

Launch:  12-09-92  14:23:00 UTC

If the satellite to be tracked is not listed in modes.dat or if no beacon

frequency is specified, the program will use a default value of 146.000

MHz. Note that if a satellite is listed in modes.dat its name in the line

'Satellite:' has to exactly match the name in the NORAD two-element data

file with lower and upper case letters. For example, AO-10 has to be

entered as OSCAR 10 and AO-21 as INFORMTR-1. An alias name can be

specified in the 'Alias:' line. This name will be used in addition to the

NORAD satellite name for the live display and the prediction tables. If a

launch date is specified in modes.dat the live display will show the

mission elapsed time (MET).

PROGRAM START

To start the program type 'sattrack [-t] [-v]'. The options are as follows:

-t   allows specification of another time zone instead of the default time

zone listed in defaults.dat. This is useful to quickly run off a

prediction for a site elsewhere in the world without having to change

the default data file.

-v   enables the verbose mode. All default parameters, ground station data

and detailed orbital elements will be displayed at program start.

For all keyboard entries the default parameters are shown in <>. Hitting

the RETURN key will take these. Both lower and upper case letters will be

accepted. In case of the site and satellite name a fraction of the name

will be sufficient. The program will take the first site or satellite name

that matches the character string. One can choose between a live display

and an orbit prediction in a short and a long form. Examples of these are

given below.

LIVE DISPLAY

Once in the live display mode, the program cycles through a full

computation every 5 seconds and sleeps in between. In a strict sense,

the live display is not a real-time display since it runs under UNIX.

For most applications this is not a disadvantage. However, it is very

important that the UNIX system clock is set correctly. The live display is

rather self-explanatory. An example of the screen layout is shown below.

In case of the space shuttle, the computation of the orbit number is

different from all other satellites. For the shuttle, the orbit

number increments each time the satellite crosses the ascending node.

The shuttle is launched into orbit #1. If the space shuttle is tracked,

the source of the element set, i.e. the element set type has to be

specified, since there is a difference in the convention for definition

of the reference orbit between NASA and NORAD. For satellites other than

the space shuttle the NORAD convention is used and the orbit number

increments when the satellite passes through the perigee.

The visibility display to the right of the percentage of the current orbit

indicates if the satellite is in the day ('D') or night ('N') part of its

orbit. If the satellite is above the horizon and in the day part of its

orbit and at the same time the Sun is below the nautical twilight limit

(-6 deg elevation), then the display shows 'V' (visible), since the

satellite will be visible to the naked eye, at least for the larger

satellites. However, the apparent brightness depends on the relative

angles between the Sun, the satellite and the ground station, and on the

shape and size of the satellite.

The eclipse and visibility conditions are also indicated by 'D', 'N' or

'V' in the lower right corner of the display for the circumstances of the

next pass, i.e. acquisition of signal (AOS), maximum elevation (MEL) and

loss of signal (LOS) of the satellite.

Azimuth and elevation angles of the Sun are shown for the location of the

ground station.

The Doppler shift is calculated for the beacon frequency displayed one

line above. The free-space path loss can be used for link quality

calculations.

The satellite mode depends on the phase of the satellite (the mean anomaly

in units of 0-256), as specified in modes.dat. If no satellite mode is

specified, the display shows blank.

In case of the space shuttle, distances are displayed in nautical miles

(NM) and otherwise they are in kilometers. Accordingly, velocities are

in NM/s or km/s.

The state vector of the satellite (position and velocity in cartesian

coordinates) is displayed below the bearing and ground track information.

The circumstances of the next pass are calculated and displayed in the

lower part of the screen. If the next pass has a maximum elevation of more

than 40 deg the maximum elevation and range at maximum elevation are shown

in reverse video. The countdown time represents the time until next AOS or

next LOS. One minute before AOS and during a satellite pass the countdown

time is displayed in reverse video.

The mission elapsed time (MET) will be displayed if the launch date and

time are specified in modes.dat.

The groundtrack location indicates the location of the sub-satellite

point with respect to the nearest location in the city data base. For now,

there is only one fixed location (Kennedy Space Center) hardwired in the

code, since an appropriate city data base could not be obtained so far.

An alarm (beep) is generated for each screen update less than one minute

away from either AOS or LOS. When the satellite being tracked is above the

horizon several numbers on the screen show up in reverse (the header line,

azimuth, elevation, range, Doppler shift and path loss).

Once the program is running in the live display mode, it can be frozen

and unfrozen with ^S and ^Q, and aborted with ^C. Hitting the <SPACE>

bar will restart the program.

ORBIT PREDICTION

The orbit prediction mode allows the calculation of satellite passes for

a given ground station. The data will be either displayed on the video

terminal or written into a file in the Prediction directory which can

later be obtained as a hardcopy. The program generates tables in a short

and in a long form. The short form listing consists of one table per day

with one line per pass, indicating the time at AOS, MEL and LOS, the

duration of the pass, the azimuth at AOS, MEL and LOS, the maximum

elevation of the pass, the visibility code for AOS, MEL and LOS, and

number of the orbit in which the satellite is at AOS. Also, if a launch

date is specified in 'modes.dat' an additional column shows the MET at

AOS. Passes with a maximum elevation of more than 40 deg are marked with

an asterisk next to the MEL value.

The prediction in the long format consists of 12 columns. The first column

is the time in specified fixed increments from line to line. Most of the

table is self-explanatory. The column labeled 'Loss' is the free-space

path loss in dB. The column labeled 'Phs' is the orbital phase of the

satellite, i.e. the mean anomaly running from 0 through 256. The 'Md'

column is the satellite mode, derived from the phase and the entries in

modes.dat. This column will be blank if no modes are specified in

modes.dat. The last column is the visibility code of the satellite ('D',

'N' or 'V', see above).

An example for a prediction table in the short and long form is given

below.

BUGS AND IMPERFECTIONS

There is a number of inconveniences or imperfections that have not yet

been fixed. These will be taken care off in a later version.

1.   The 'nearest city' display is not yet implemented. It uses only one

location (KSC) at this time.

2.   The spacecraft's attitude can be specified in 'modes.dat', but is not

taken into account yet for the calculation of the squint angle.

3.   The calculation of the height of the satellite does not take into

account the geoid model of the Earth.

4.   The azimuth and elevation angles of the Sun are precise within

0.15 deg only.

AUXILIARY PROGRAMS

The following two programs are not used with SatTrack but are rather

useful for other purposes.

amsat:        generates a data file 'amsat.dat' in SatTrack/Data with

orbital elements in the AMSAT format from a data file with

orbital elements in the NORAD two-line format.

readelements: reads two-line elements from a NORAD data file and displays

them in plain language.

Example of the live display:

SatTrack____________________STS-45_TRACKING_MONITOR______________________DL5KR

Ground Stn :    Berkeley, CA    Date: 27-03-92    Radio Beacon :  145.5500 MHz

Satellite  :        Atlantis    Day : 087--Fri    Doppler Shift:   +1.0119 kHz

Inclination:      56.993 deg    UTC : 22:57:59    Path Loss    :  126.8000 dB

Orbit      :    72  37.0 % D    PST : 14:57:59    Phase (0-256):  147.6900

Sun Azi/Ele: 195.0  40.0 deg                      Mode  (ABJLS):        FM

Azimuth   :      267.153 deg            Latitude  N  :   +37.612 deg

Elevation :       61.409 deg            Longitude W  :  +124.043 deg

Range     :      187.995 NM             Height       :   162.093 NM

Range Rate:       -1.125 NM/s           Velocity     :     4.170 NM/s

State Vector X:  +1981.910 NM        Y:  +2057.186 NM        Z:  +2200.783 NM

VX:     -0.783 NM/s     VY:     +3.319 NM/s     VZ:     -2.400 NM/s

Last AOS  :  87/14:54:13 PST            AOS Azimuth  :   315.112 deg  D

Duration  :   0/00:08:30                MEL Azimuth  :   232.474 deg  D

Next LOS  :  87/15:02:43 PST            LOS Azimuth  :   144.161 deg  D

Countdown :   0/00:04:44                Max Elevation:    66.566 deg

MET       :   4/08:30:25                MEL Range    :   406.270 NM

Groundtrack Location:  2000.0 NM   W of Kennedy Space Center, FL

Example of a prediction table in short form:

DL5KR SatTrack Prediction

Satellite         : STS-47 (Endeavour)

NORAD Data File   : STS-47

Element Set       : 6

Element Set Type  : NORAD

Epoch             : 12-09-92  20:21:31.000 UTC

Ground Station    : Berkeley CA

Time Zone         : -7 (PDT)

Date (PDT)          Time (PDT) at        Duration   Azimuth at   Max  Vis Orbit

AOS      MEL      LOS     of Pass  AOS MEL LOS  Elev

Sun 13-09-92  15:26:44 15:27:39 15:28:22  00:01:38    5  16  25   0.2  DDD    22

16:58:43 17:02:34 17:06:33  00:07:50  337  33  93  10.9  DDD    23

18:32:09 18:36:40 18:41:15  00:09:06  312 234 150  45.0* DDD    24

Mon 14-09-92  09:14:35 09:18:39 09:22:58  00:08:23  190 126  58  17.4  DDD    34

10:48:08 10:52:27 10:56:50  00:08:42  250 320  30  20.2  DDD    35

12:25:06 12:27:08 12:29:27  00:04:21  311 337   7   2.1  DDD    36

15:36:10 15:38:26 15:40:50  00:04:40  352  22  53   2.4  DDD    38

17:08:50 17:13:08 17:17:37  00:08:47  330  37 113  22.1  DDD    39

18:42:45 18:46:50 18:51:03  00:08:17  301 239 172  15.8  DDD    40

Tue 15-09-92  07:55:26 07:55:53 07:56:20  00:00:54  120 115 109   0.1  DDD    49

09:24:25 09:28:57 09:33:31  00:09:06  212 124  46  51.3* DDD    50

10:59:12 11:03:03 11:06:52  00:07:39  270 327  22  10.1  DDD    51

12:37:48 12:38:01 12:38:31  00:00:43  342 345 351   0.0  DDD    52

15:45:58 15:49:08 15:52:32  00:06:34  344  27  75   5.9  DDD    54

17:18:59 17:23:31 17:28:13  00:09:15  321  33 132  58.3* DDD    55

18:53:51 18:57:02 19:00:04  00:06:12  284 240 198   5.4  DDD    56

Wed 16-09-92  08:02:27 08:05:51 08:09:10  00:06:42  167 118  72   6.9  DDD    65

09:34:37 09:39:09 09:43:46  00:09:09  232 315  37  46.0* DDD    66

11:10:32 11:13:42 11:16:44  00:06:12  290 334  15   4.9  DDD    67

14:23:44 14:24:38 14:25:46  00:02:02    3  14  29   0.4  DDD    69

15:55:48 15:59:53 16:03:46  00:07:58  336  37  95  11.7  DDD    70

17:29:13 17:33:45 17:38:20  00:09:06  311 236 152  40.0* DDD    71

Example of a prediction table in long form:

DL5KR SatTrack Prediction

Satellite         : STS-47 (Endeavour)

NORAD Data File   : STS-47

Element Set       : 6

Element Set Type  : NORAD

Epoch             : 12-09-92  20:21:31.000 UTC

Ground Station    : Berkeley CA

Time Zone         : -7 (PDT)

Min. Elevation    : 10.0 deg

Doppler Reference : 145.5500 MHz

Sun 13-09-92  ---  Orbit 24  ---  MET:   0/00:00:00

PDT   Azimuth  Elev   Range    Lat    Long    Height  Doppler  Loss  Phs Md V

[deg]  [deg]    [NM]   [deg]   [deg]    [NM]    [kHz]   [dB]

18:34:26  304.3   12.6    584.4  +42.4  +132.4    167.7   +3.23  136.4  144 FM D

18:34:56  299.9   17.4    479.1  +41.0  +130.6    167.7   +3.07  134.7  146 FM D

18:35:26  292.7   24.1    381.0  +39.6  +128.9    167.6   +2.77  132.7  147 FM D

18:35:56  279.3   33.3    297.7  +38.2  +127.2    167.6   +2.15  130.5  148 FM D

18:36:26  252.4   43.1    244.7  +36.8  +125.6    167.5   +0.91  128.8  150 FM D

18:36:56  212.6   43.4    242.8  +35.3  +124.1    167.4   -0.80  128.8  151 FM D

18:37:26  184.7   33.7    293.1  +33.8  +122.6    167.4   -2.09  130.4  153 FM D

18:37:56  170.8   24.3    375.1  +32.3  +121.2    167.3   -2.74  132.5  154 FM D

18:38:26  163.3   17.5    472.5  +30.8  +119.8    167.2   -3.06  134.5  156 FM D

18:38:56  158.8   12.5    577.5  +29.3  +118.5    167.2   -3.22  136.3  157 FM D

Mon 14-09-92 --- Orbit 34

PDT   Azimuth  Elev   Range    Lat    Long    Height  Doppler  Loss  Phs Md V

[deg]  [deg]    [NM]   [deg]   [deg]    [NM]    [kHz]   [dB]

09:17:06  165.2   11.3    606.0  +28.5  +119.5    165.7   +2.22  136.7   78 FM D

09:17:36  155.2   14.0    539.7  +30.1  +118.2    165.8   +1.72  135.7   79 FM D

09:18:06  142.6   16.2    493.1  +31.6  +116.9    165.9   +1.04  134.9   81 FM D

09:18:36  127.8   17.4    472.0  +33.1  +115.5    166.0   +0.20  134.5   82 FM D

09:19:06  112.6   17.0    480.0  +34.6  +114.0    166.1   -0.67  134.7   84 FM D

09:19:36   98.8   15.3    515.6  +36.1  +112.5    166.3   -1.44  135.3   85 FM D

09:20:06   87.6   12.8    573.6  +37.5  +111.0    166.4   -2.02  136.2   86 FM D

09:20:36   78.9   10.1    648.1  +39.0  +109.4    166.5   -2.43  137.3   88 FM D