The Satellite Tracking Program SatTrack

=============================================

Manfred Bester, DL5KR

December 13, 1992

SYNOPSIS

SatTrack is a satellite tracking program that has been written in 'C'

code on a Sun/Unix workstation in order to provide an alternative to PC

oriented programs like InstantTrack or QuikTrak. SatTrack is based on

program 'Orbit' by Robert W. Berger (N3EMO), but has been completely

re-written and enhanced with many new features. It provides a live display

which Orbit does not have, and other orbit prediction facilities, but no

graphics either. Cursor controls used in the live display are compatible

with the VT100 standard. The program reads the NORAD two-line element sets

directly. Other input files are defaults.dat, sites.dat, cities.dat and

modes.dat. These contain information on default parameters and various

sites and satellite modes like beacon frequencies, launch dates,

attitudes, transmission modes and so on. The file cities.dat contains the

data base for the nearest city display.

INSTALLATION

To install SatTrack on a Sun workstation, create the following

directories in your Unix home directory:

SatTrack/Data

SatTrack/Doc

SatTrack/Object

SatTrack/Prediction

SatTrack/Run

SatTrack/Source

Move all source code into SatTrack/Source. Move all auxiliary data

files (see below) into SatTrack/Data. Edit the file sattrack.h and change

the variable SATUSER to your login name. Also edit the Makefile in

SatTrack/Source and change the USER variable to you login name or home

directory. Change to directory SatTrack/Source and type 'make sattrack'.

Edit your .login and/or .cshrc files to include ~/SatTrack/Run in your

path.

The Run directory will contain the executable files and the Object

directory the object code. The Data directory contains the files

with the NORAD two-line element sets as well as defaults.dat, modes.dat,

cities.dat and sites.dat. Predictions can be written into an output file

which will be placed in the Prediction directory. The Doc directory will

contain all documentation on SatTrack.

INPUT DATA FILES

Program SatTrack requires five input data files:

tle.dat            file with NORAD two-line element sets

defaults.dat       contains default start-up parameters

sites.dat          contains the data base with ground station parameters

(name, latitude, longitude, altitude)

cities.dat         contains data on nnn cities and other special locations

(this part is presently under construction; one location

is presently hardwired in satcity.c)

modes.dat          contains additional information on satellites

(beacon frequencies, modes, launch dates etc.)

Example of tle.dat:

- Current Two-Line Element Sets #053 -

Mir

1 16609U 86 17  A 92253.83202629  .00010821  00000-0  17200-3 0  5392

2 16609  51.6235 184.3926 0003511 132.9676 227.1601 15.53877561375443

COBE

1 20322U 89 89  A 92253.56214384 -.00000059  00000-0 -26243-4 0  5574

2 20322  99.0083 266.1778 0008361 270.9185  89.1022 14.03352237143838

PACSAT

1 20439U 90  5  D 92246.03439168  .00000070  00000-0  35288-4 0  4890

2 20439  98.6417 327.8611 0011066 315.7837  44.2460 14.29731036136265

DO-17

1 20440U 90  5  E 92251.71853570  .00000070  00000-0  34955-4 0  4902

2 20440  98.6415 333.6363 0010985 298.5546  61.4530 14.29859354137082

TDRS 4

1 21639U 91 54  B 92249.52615843  .00000111  00000-0  00000+0 0  2232

2 21639   0.0810 282.2753 0002729 241.6567 196.0869  1.00275770  2147

EUVE

1 21987U 92 31  A 92232.33773712  .00002181  00000-0  10297-3 0    77

2 21987  28.4322 199.3801 0010738 262.4472  97.5038 15.15165949 11029

STS-47

1 22120U 92 61  A 92259.61569380 0.00076435  00000-0  25599-3 0   124

2 22120  56.9971  93.6720 0008801 289.1809  70.8314 15.89530661   497

The meaning of the numbers in each line are explained in the source

code listing (see 'readnorad.c'). Elements sets like the above can be

obtained from 'archive.afit.af.mil' (129.92.1.66) via anonymous FTP.

Change into the directory '/pub/space' and get 'tle.new'. Then place this

file as 'tle.dat' in the Data directory. Alternatively, the element sets

can be obtained from the Celestial Bulletin Board System in Fairborn,

Ohio, via modem line (513) 427-0674. Latest element sets for space shuttle

missions can be obtained from 'spacelink.msfc.nasa.gov' (192.149.89.61)

via TELNET or via telephone by dialing (205) 895-0028.

Example of defaults.dat:

Site:             Berkeley, CA

Satellite:        Mir

Element set:      tle.dat

Element set type: NORAD

Time zone:        PST

Duration:         3.0

Min elevation:    10.0

These parameters are the defaults that show up in '<>' brackets when

starting program 'sattrack'.

Example of sites.dat:

Auckland, NZL            -36.000000 -174.500000     0.0

Berkeley, CA              37.873931  122.240372   400.0

KSC, FL                   28.475000   80.610000     5.0

Edwards AFB               34.940000  117.860000   200.0

Guam                      13.330000 -144.720000     5.0

Haleakala AMOS            20.708333  156.257500  3052.0

Honolulu, HI              21.310000  157.850000     0.0

Houston JSC               29.875000   95.420000    50.0

Mauna Kea, HI             19.826667  155.471667  4215.0

Midway Island             28.360000  177.380000     5.0

Moscow                    55.700000  -37.545000   195.0

Rio De Janeiro           -22.895000   43.223333    33.0

Washington, DC            38.921667   77.066667    92.0

The length of the name of the ground station has to be 17 characters or

less (otherwise it will be truncated to a length of 17 characters).

The first column of numbers is the geographical latitude [deg North],

the second the geographical longitude [deg West], and the third the

altitude [m]. Note that some other tracking programs count the longitude

in eastern direction. Here the convention of western longitudes being

positive is used.

Example of cities.dat:

--- not yet required --- (still looking for a city data base)

Example of modes.dat:

Satellite: OSCAR 10

Alias: AO-10

Beacon:          145.810000 MHz

Mode: B           000 - 256

Satellite: AO-13

Beacon:          145.812000 MHz

Attitude:        180.0  0.0 deg

Mode: B           000 - 100

Mode: JL          100 - 120

Mode: LS          120 - 135

Mode: JL          135 - 150

Mode: B           150 - 256

Satellite: PACSAT

Alias: AO-16

Beacon:          437.026250 MHz

Satellite: INFORMTR-1

Alias: AO-21

Beacon:          145.822000 MHz

Satellite: Mir

Beacon:          145.550000 MHz

Mode: FM            0 - 256

Satellite: STS-47

Alias: Endeavour

Beacon:          145.550000 MHz

Mode: FM            0 - 256

Launch:  12-09-92  14:23:00 UTC

If the satellite to be tracked is not listed in modes.dat or if no beacon

frequency is specified, the program will use the default value of

146.000 MHz. Note that if a satellite is listed in modes.dat its name in

the line 'Satellite:' has to exactly match the name in the NORAD

two-element data file with lower and upper case letters. For example,

AO-10 has to be entered as OSCAR 10 and AO-21 as INFORMTR-1. An alias name

can be specified in the 'Alias:' line. This name will be used in addition

to the NORAD satellite name for the live display and the predictions.

If a launch date is specified in modes.dat the live display will show the

mission elapsed time (MET).

PROGRAM START

To start the program type 'sattrack [-t] [-v]'. The options as follows:

-t   allows specification of a time zone instead of the default time

zone listed in defaults.dat. This is useful to quickly run off

a prediction for a site elsewhere in the world without having to

change the default data file.

-v   enables the verbose mode; default parameters and detailed orbital

elements will be displayed at program start.

For all keyboard entries the default parameters are shown in <>.

Hitting the RETURN key will take these. Both lower and upper case letters

will be accepted. In case of the site and satellite name a fraction of the

name will be sufficient. The program will take the first site or satellite

name that matches the character string. One can choose between a live

display and an orbit prediction in a short and a long form.

LIVE DISPLAY

The live display is rather self-explanatory. An example of the screen

layout is shown below.

In case of the space shuttle, the computation of the orbit number is

different from all other satellites. For the shuttle, the orbit

number increments each time the satellite crosses the ascending node.

The shuttle is launched into orbit #1. If the space shuttle is tracked,

the source of the element set, i.e. the element set type has to be

specified, since there is a difference in the convention for definition

of the reference orbit between NASA and NORAD. For satellites other than

the space shuttle the NORAD convention is used and the orbit number

increments when the satellite passes through the perigee.

The visibility display to the right of the percentage of the current orbit

indicates if the satellite is in the day ('D') or night ('N') part of its

orbit. If the satellite is above the horizon and in the day part of its

orbit and at the same time the Sun is below the nautical twilight limit

(-6 degrees elevation), then the display shows 'V' (visible), since the

satellite will be visible to the naked eye (at least for the larger

satellites). This condition is also indicated by 'D', 'N' or 'V' in the

lower right corner of the display for the circumstances of the next

pass, i.e. rise, culmination (i.e. closest range or maximum elevation) and

set of the satellite.

Azimuth and elevation angles of the Sun are shown for the location of the

ground station.

The Doppler shift is calculated for the beacon frequency displayed one

line above. The free-space path loss is displayed only when the satellite

being tracked is above the horizon.

The satellite mode depends on the phase (in units of 0-256), as specified

in modes.dat. If no satellite mode is specified, the display shows blank.

In case of the space shuttle, length units displayed are nautical miles

(NM) and otherwise they are kilometers.

The state vector (position and velocity in cartesian coordinates) is

displayed below the bearing and ground track information.

The circumstances of the next pass are calculated and displayed in the

lower part of the screen. The MET (Mission Elapsed Time) will be displayed

if the launch date and time are specified in modes.dat.

The Ground Track Location indicates the location of the sub-satellite

point with respect to the nearest location in the city data base.

Once the program is running in the live display mode, it can be frozen

with ^S and ^Q, and aborted with ^C.

ORBIT PREDICTION

The orbit prediction mode allows the calculation of satellite passes for

a given ground station. The data will be either displayed on the video

terminal or written into a file in the Prediction directory which can

later be obtained as a hardcopy. If the program is run by another user the

prediction will be written into a file in that user's home directory.

The program generates tables in a short and in a long form. The short form

listing consists of one table per day with one line per pass, indicating

the time at AOS, maximum elevation (MEL) and LOS, the azimuth at AOS, MEL

and LOS, the maximum elevation of the pass, the visibility code for AOS,

MEL and LOS, and the orbit number.

The prediction in the long format consists of 12 columns, where the

first one is the time in fixed increments from line to line.

Most of it is self-explanatory. The column labeled 'Loss' is the free-

space path loss in dB. The column labeled 'Phs' is the orbital phase of

the satellite running from 0 through 256. The 'Md' column is the satellite

mode, derived from the phase and the entries in modes.dat. This column

will be blank if no modes are specified in modes.dat. The last column is

the visibility of the satellite (see above).

An example for a prediction table in short and long form is given below.

AUXILIARY PROGRAMS

The following two programs are not used with SatTrack but are rather

useful for other purposes.

amsat:        generates a data file with orbital elements in the AMSAT

format from a data file with orbital elements in the NORAD

two-line format.

readelements: reads two-line elements from a NORAD data file and displays

them in plain language.

STS-45 TRACKING MONITOR

Station    :    Berkeley SSL    Date: 27-03-92    Radio Beacon :  145.5500 MHz

Satellite  :          STS-45    Day : 087--Fri    Doppler Shift:   +1.0119 kHz

Orbit      :    72  37.0 % D    UTC : 22:57:59    Path Loss    :  126.8000 dB

Sun Az/Elev: 195.0  40.0 deg    PST : 14:57:59    Phase (0-256):  147.6900

Mode  (ABJLS):        FM

Azimuth   :      267.153 deg            Latitude  N  :   +37.612 deg

Elevation :       61.409 deg            Longitude W  :  +124.043 deg

Range     :      187.995 NM             Height       :   162.093 NM

Range Rate:       -1.125 NM/s           Velocity     :     4.170 NM/s

State Vector X:  +1981.910 NM        Y:  +2057.186 NM        Z:  +2200.783 NM

VX:     -0.783 NM/s     VY:     +3.319 NM/s     VZ:     -2.400 NM/s

Last AOS  :  87/14:54:13 PST            AOS Azimuth  :   315.112 deg  D

Duration  :   0/00:08:30                MEL Azimuth  :   232.474 deg  D

Next LOS  :  87/15:02:43 PST            LOS Azimuth  :   144.161 deg  D

Countdown :   0/00:04:44                Max Elevation:    66.566 deg

MET       :   4/08:30:25                MEL Range    :   406.270 NM

Ground Track Location: _____._ NM NNE of Kennedy Space Center, FL

DL5KR SatTrack Prediction

Satellite         : STS-47 (Endeavour)

NORAD Data File   : STS-47

Element Set       : 6

Element Set Type  : NORAD

Epoch             : 12-09-92  20:21:31 UTC

Ground Station    : Berkeley, CA

Time Zone         : -7 (PDT)

Date (PDT)          Time (PDT) at        Duration   Azimuth at   Max  Vis Orbit

AOS      MEL      LOS     of Pass  AOS MEL LOS  Elev

Sun 13-09-92  15:26:44 15:27:39 15:28:22  00:01:38    5  16  25   0.2  DDD    22

16:58:43 17:02:34 17:06:33  00:07:50  337  33  93  10.9  DDD    23

18:32:09 18:36:40 18:41:15  00:09:06  312 234 150  45.0  DDD    24

Mon 14-09-92  09:14:35 09:18:39 09:22:58  00:08:23  190 126  58  17.4  DDD    34

10:48:08 10:52:27 10:56:50  00:08:42  250 320  30  20.2  DDD    35

12:25:06 12:27:08 12:29:27  00:04:21  311 337   7   2.1  DDD    36

15:36:10 15:38:26 15:40:50  00:04:40  352  22  53   2.4  DDD    38

17:08:50 17:13:08 17:17:37  00:08:47  330  37 113  22.1  DDD    39

18:42:45 18:46:50 18:51:03  00:08:17  301 239 172  15.8  DDD    40

Tue 15-09-92  07:55:26 07:55:53 07:56:20  00:00:54  120 115 109   0.1  DDD    49

09:24:25 09:28:57 09:33:31  00:09:06  212 124  46  51.3  DDD    50

10:59:12 11:03:03 11:06:52  00:07:39  270 327  22  10.1  DDD    51

12:37:48 12:38:01 12:38:31  00:00:43  342 345 351   0.0  DDD    52

15:45:58 15:49:08 15:52:32  00:06:34  344  27  75   5.9  DDD    54

17:18:59 17:23:31 17:28:13  00:09:15  321  33 132  58.3  DDD    55

18:53:51 18:57:02 19:00:04  00:06:12  284 240 198   5.4  DDD    56

Wed 16-09-92  08:02:27 08:05:51 08:09:10  00:06:42  167 118  72   6.9  DDD    65

09:34:37 09:39:09 09:43:46  00:09:09  232 315  37  46.0  DDD    66

11:10:32 11:13:42 11:16:44  00:06:12  290 334  15   4.9  DDD    67

14:23:44 14:24:38 14:25:46  00:02:02    3  14  29   0.4  DDD    69

15:55:48 15:59:53 16:03:46  00:07:58  336  37  95  11.7  DDD    70

17:29:13 17:33:45 17:38:20  00:09:06  311 236 152  40.0  DDD    71

DL5KR SatTrack Prediction

Satellite         : STS-47 (Endeavour)

NORAD Data File   : STS-47

Element Set       : 6

Element Set Type  : NORAD

Epoch             : 12-09-92  20:21:31 UTC

Ground Station    : Berkeley, CA

Time Zone         : -7 (PDT)

Min. Elevation    : 10.0 deg

Doppler Reference : 145.5500 MHz

Sun 13-09-92 --- Orbit 24

PDT   Azimuth  Elev   Range    Lat    Long    Height  Doppler  Loss  Phs Md V

[deg]  [deg]    [NM]   [deg]   [deg]    [NM]    [kHz]   [dB]

18:34:26  304.3   12.6    584.4  +42.4  +132.4    167.7   +3.23  136.4  144 FM D

18:34:56  299.9   17.4    479.1  +41.0  +130.6    167.7   +3.07  134.7  146 FM D

18:35:26  292.7   24.1    381.0  +39.6  +128.9    167.6   +2.77  132.7  147 FM D

18:35:56  279.3   33.3    297.7  +38.2  +127.2    167.6   +2.15  130.5  148 FM D

18:36:26  252.4   43.1    244.7  +36.8  +125.6    167.5   +0.91  128.8  150 FM D

18:36:56  212.6   43.4    242.8  +35.3  +124.1    167.4   -0.80  128.8  151 FM D

18:37:26  184.7   33.7    293.1  +33.8  +122.6    167.4   -2.09  130.4  153 FM D

18:37:56  170.8   24.3    375.1  +32.3  +121.2    167.3   -2.74  132.5  154 FM D

18:38:26  163.3   17.5    472.5  +30.8  +119.8    167.2   -3.06  134.5  156 FM D

18:38:56  158.8   12.5    577.5  +29.3  +118.5    167.2   -3.22  136.3  157 FM D

Mon 14-09-92 --- Orbit 34

PDT   Azimuth  Elev   Range    Lat    Long    Height  Doppler  Loss  Phs Md V

[deg]  [deg]    [NM]   [deg]   [deg]    [NM]    [kHz]   [dB]

09:17:06  165.2   11.3    606.0  +28.5  +119.5    165.7   +2.22  136.7   78 FM D

09:17:36  155.2   14.0    539.7  +30.1  +118.2    165.8   +1.72  135.7   79 FM D

09:18:06  142.6   16.2    493.1  +31.6  +116.9    165.9   +1.04  134.9   81 FM D

09:18:36  127.8   17.4    472.0  +33.1  +115.5    166.0   +0.20  134.5   82 FM D

09:19:06  112.6   17.0    480.0  +34.6  +114.0    166.1   -0.67  134.7   84 FM D

09:19:36   98.8   15.3    515.6  +36.1  +112.5    166.3   -1.44  135.3   85 FM D

09:20:06   87.6   12.8    573.6  +37.5  +111.0    166.4   -2.02  136.2   86 FM D

09:20:36   78.9   10.1    648.1  +39.0  +109.4    166.5   -2.43  137.3   88 FM D